# Bertha Shot Sheet 010926

Notes:

- MCP camera uncovered

- Lots of plasma ablating off electrode surfaces- reshape electrodes

- Polarizers not set

- Rogowski signal weak (¼ posts)

- Faraday2 cam needs focusing

- Need to set up interferometry

# Scope

- DSO1

  - Master trigger

  - Current return rogowski

  - MCP monitor

  - Laser diode

# Shot

- Supply Voltage at shot time: 46 kV