# Bertha Shot Sheet 030426

# Notes:

- Prefire at 46 kV, no data collected

- Strong interferometry contrast pre-shot due to use of diode seed laser (previously turned off)

- Remounted and focused MCP, covered with cloth

- Faraday blocked off for now until realignment with new camera is done

- Problems with trigger preshot, eventually resolved (with very strong arcs on 3-4 electrodes!) but still unsure of cause/reason for resolution

  - Tested delay generators, both are fine

  - Tested optical transmitters with larger pulse width utilizing current protection resistor (50 ohm) and confirmed that most are working properly

    - Original bertha transmitter is shot, second transmitter on rama box is shot, fourth transmitter on rama box is intermittent and weak, and all 4 others are fine

  - Trigger worked properly with longer (10 ms) pulse width and current protection resistor, then started working as expected without the resistor and the original 1 us pulse width

- Trigger also sending high voltage pulse back along supply cables, causing spark near batteries and sometimes another unidentified place near the control tower

# Optical Diagnostics

- Mach-Zender Interferometer

- MCP

# Scope

- DSO1

  - Master trigger

  - Current return rogowski

  - MCP monitor

  - Laser diode

# Shot

- Supply Voltage at shot time: 46 kV