# Bertha Shot Sheet 030426

# Notes:

- Machine seemed to fire fine at 45kV, scope triggered but no images or rogowski trace. Laser, MCP, and master trigger all visible on trace.

- Trying to find location of trigger supply cable breakdown

  - Happens at solder joint on floor, jumps to shielding on some coax cable

    - Issue has been resolved with kapton tape on battery cable

    - Also happens on supply battery body, stopped happening after moving batteries on top of plastic housing

  - Can still hear it happening somewhere else behind control tower

    - Could be caused by laser focusing after the negative lens instead of another trigger supply arc, fine as long as it isn’t on the lens itself

  - Tried to diagnose by setting delay higher, moving around to back, but realized that this setup causes the system to keep triggering at a regular interval despite only one input

    - Is noise generated by the spark getting picked up on the inhibit or main trigger cable causing the system to trigger itself in a loop?

      - Don’t think it’s the inhibit because the first delay generator doesn’t show that it’s being triggered

      - Could easily be noise on main trigger; we see this noise every shot and it exceeds the actual signal amplitude (~50V peak-to-peak)

        - For now, setting scope range to 8v and clipping signal so we can clearly see master trigger

      - Might be that this only happens when setting the delay higher (5s vs 1 us) as otherwise the ‘parasitic trigger’ occurs before the trigger box can recover and pulse again, so it’s not a problem for actual shots

- Noise from trigger is significant on laser diode channel

  - Initially worried similar noise on q-switch cable could prematurely fire laser, messing up timing

    - After shielding, doesn’t seem to be a consistent problem anymore, though noise is still very apparent

  - Faraday cage returned to laser power supply

  - Flashlamp cable shielding added as the cables used were unshielded

# Optical Diagnostics

- Mach-Zender Interferometer

- MCP

# Scope

- DSO1

  - Master trigger

  - Current return rogowski

  - MCP monitor

  - Laser diode

# Shot

- Supply Voltage at shot time: 45 kV